# Flat Handover Checklist

App content, draft 1 for your review. 26 September 2026.

This is the content the app will show a buyer on handover day. It is written for ordinary buyers, not engineers: every check can be done in one visit with simple tools. Please correct anything that does not match Bangladeshi practice or your experience.

# How to review this draft

- Edit the text directly, or write in the Your note column. Delete what you do not want and add what is missing.

- Items marked [Confirm] need your professional check, such as a code figure or local practice. There are six: A21, B13, D2, E3, E10 and G8.

- Keep each How to check entry short, because it has to fit on a phone screen.

- Use the item codes (A1, B4 and so on) to refer to items in chat.

# How the app will use this list

The buyer first enters the flat layout (number of bedrooms, bathrooms and balconies). The app then builds a room-by-room list from the sections in this document:

| Room or area | Sections used |
| --- | --- |
| Living room, dining room, each bedroom | A |
| Each bathroom | A and B |
| Kitchen | A and C |
| Each balcony | D, plus the wall and ceiling checks A1 to A6 |
| Whole flat, once | E |
| Building and common areas, once | F |
| End of the visit | G |

For each item the buyer taps OK, Problem or Not applicable. Every group has an All OK button, so a clean room takes about a minute. For a problem, the buyer picks a severity level, adds a short note and takes photos. The report lists problems room by room with Safety items first, then a short list of everything checked OK as a record, and is saved as a PDF to share with the developer.

# Severity levels

| Level | Meaning | Examples |
| --- | --- | --- |
| Minor | Looks wrong but does not affect use. | Paint stains, small chips, a loose switch plate, uneven grout colour. |
| Major | Affects use, or will get worse with time. | Leaks, damp patches, hollow tiles, doors or windows that do not close, water pooling on the floor. |
| Safety | Can hurt someone. Must be fixed before moving in. | Exposed wires, sparking or hot sockets, loose railings, gas smell, wide or diagonal cracks in beams, columns or slabs. |

# Before you go (start screen)

## What to bring

- Your charged phone with this app (camera and torch)

- A coin, or a small rubber mallet, for tapping tiles and plaster

- A phone charger, to test sockets

- A marble or small ball, to check floor slope

- A bucket for drain tests, or ask the site staff for water

- A 5 m measuring tape

- Masking tape and a marker, to mark each defect on site before photographing it

- Your sale agreement, specification list and floor plan

## Ground rules

- Go in daylight, and ask the developer to keep electricity, water and the lift running during your visit.

- Take someone with you, ideally an engineer.

- Do not open the distribution board (DB) cover or touch bare wires.

- Check for gas leaks only with soapy water, never with a flame.

- Test railings and grills by pushing with your hands while standing firmly. Never lean your body over them.

# A. Every room

Used for the living room, dining room, bedrooms, kitchen and bathrooms. Anything a room does not have (for example no window) is marked Not applicable with one tap.

## Walls and ceiling

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| A1 | Wall cracks | Scan each wall in side light, especially around door and window corners. Mark Major if a coin edge fits in or the crack runs diagonally. |  |
| A2 | Cracks in beams, columns or slab | A thin line where a brick wall meets a beam or column is common. A crack across a beam, column or slab: mark Safety and have an engineer see it. |  |
| A3 | Damp patches or white salt marks | Look for dark stains, bubbling paint, white powdery patches or a musty smell, especially on outside walls and walls shared with bathrooms. |  |
| A4 | Hollow plaster | Knock on each wall at a few points with a knuckle or the mallet. A hollow or dull sound means the plaster has come away from the wall. |  |
| A5 | Rust stains or flaking concrete on the ceiling | Brown streaks, bulging or flaking patches can mean the steel inside the slab is rusting. Mark Safety and have an engineer see it. |  |
| A6 | Paint and wall finish | Check for patches, drips and uneven colour, and look along each wall at a low angle for bulges. Paint type should match the agreement. |  |

## Floor

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| A7 | Hollow tiles | Tap tiles across the room, at centres and corners, with a coin or the mallet. A hollow sound means the tile may crack or lift later. |  |
| A8 | Cracked or chipped tiles | Look closely along tile surfaces and edges, including under doors and along the skirting. |  |
| A9 | Uneven edges and grout | Slide your palm across the joints: no tile edge should stand higher than its neighbour. Joints should be fully filled, with no gaps or cracks. |  |
| A10 | Floor level | Place a marble at a few points. If it keeps rolling one way, the floor slopes; note where and in which direction. |  |
| A11 | Tile type and shade | Size, finish and brand match the agreement, with no obvious colour difference between batches. |  |
| A12 | Skirting and thresholds | Skirting firmly fixed with no hollow sound or gaps. Steps between rooms low and even, with no sharp edges. |  |

## Doors

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| A13 | Opening and closing | Open and close each door fully: no rubbing on the frame or floor, no swinging by itself, and an even gap all round. |  |
| A14 | Locks, handles and keys | Lock and unlock with every key provided. Handles should be firm. Count the keys against the handover list. |  |
| A15 | Hinges and stoppers | Hinges tight with every screw in place, and a door stopper wherever the door would hit a wall. |  |
| A16 | Door surface | Check for cracks, dents, peeling veneer and unfinished edges, including the top and bottom edges of the door. |  |

## Windows

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| A17 | Sliding and locking | Slide every shutter fully both ways. It should move smoothly without jamming, and the lock should close firmly. |  |
| A18 | Glass and rubber seals | No cracks, scratches or rattling glass. Rubber seals present and tight, with no daylight showing when the window is closed. |  |
| A19 | Rain leakage | Look for stains on the sill and the wall below. If safe, spray water on the outside and watch the inside bottom corners. |  |
| A20 | Mosquito net (if included) | Net shutters slide smoothly, and the net is tight and untorn. |  |
| A21 | Window grill | Firmly fixed, rust-free and fully painted, with bars close enough for child safety. Ask whether one grill can open in an emergency. [Confirm] |  |

## Electrical points

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| A22 | Switches | Press every switch. Each should click cleanly and control the right light or fan. |  |
| A23 | Sockets | Plug a phone charger into every socket. No power or a loose fit is Major; sparking, buzzing or heat is Safety. |  |
| A24 | Light and fan points | Every light point has power. The ceiling fan hook holds firm when pulled, and the fan regulator works. |  |
| A25 | Switch boards | Boards sit flat and firmly on the wall, with no gaps showing wires. |  |
| A26 | AC provision (bedrooms, living) | A heavy-duty AC socket, a sleeve through the wall for the pipe, a drain outlet, and a safe spot for the outdoor unit. |  |
| A27 | TV, internet and phone points | Positions match the agreement, and the conduits have a pull wire inside for future cables. |  |
| A28 | Room size and ceiling height | Measure length, width and floor-to-ceiling height, and compare with the floor plan. |  |

# B. Bathroom

Checked in each bathroom, in addition to Section A.

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| B1 | Floor slope to the drain | Pour a bucket of water on the floor. It should all run to the floor drain quickly and leave no pools. |  |
| B2 | Floor drain and smell | Grating fixed but removable for cleaning. A sewer smell means the drain trap is missing or dry: mark Major. |  |
| B3 | Tiles in the wet area | Tap wall and floor tiles around the shower for hollow spots. Joints around pipes and taps should be fully sealed. |  |
| B4 | Commode | Flush several times: it refills, stops running, does not leak at the base, and does not rock when you sit on it. |  |
| B5 | Basin | Fill and drain it. It should be firmly fixed; check underneath with a torch for drips. |  |
| B6 | Taps, shower and hand shower | Open each fully: good flow, no drips when closed, and hot and cold marked the right way round. |  |
| B7 | Water pressure | Run the shower and the basin tap together. The flow should stay steady, not drop to a trickle. |  |
| B8 | Geyser provision | A geyser power point with its own switch, hot water pipes connected to the taps, and the switch away from shower splashes. |  |
| B9 | Exhaust fan and ventilation | The exhaust fan runs quietly and pulls air; the ventilation window opens and closes. |  |
| B10 | Leaks into other rooms | Check the bathroom ceiling for stains from the flat above, and the walls of the rooms next door for damp patches. |  |
| B11 | Bathroom door | Closes fully; the bottom edge is sealed and not swollen, cracked or peeling. |  |
| B12 | Fittings | Mirror, towel rail, soap holder and other fittings in the agreement are present and firm. |  |
| B13 | Shock protection | Sockets well away from water. Ask whether the bathroom circuits have an earth-leakage breaker (RCCB). [Confirm] |  |

# C. Kitchen

Checked in the kitchen, in addition to Section A.

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| C1 | Sink and drain | Fill and drain the sink: no leaks underneath, fast drainage, no gurgling or bad smell. |  |
| C2 | Kitchen counter | Level, firmly supported and neatly finished at the edges. Tap for hollow spots under the tiles or stone. |  |
| C3 | Tiles behind stove and sink | Fully grouted with no hollow spots, and no gap where the counter meets the wall. |  |
| C4 | Gas line or LPG | Ask whether it is piped gas (Titas or another company) or LPG. If gas is connected, brush soapy water on the joints: bubbles mean a leak. Never use a flame. |  |
| C5 | Stove position | The gas point is where the stove will actually sit, with no socket or switch right beside the flame. |  |
| C6 | Kitchen exhaust | The exhaust fan or chimney point works, and the duct opening leads outside. |  |
| C7 | Appliance sockets | Heavy-duty sockets for the fridge, microwave and oven, and no socket directly above the sink or stove. |  |
| C8 | Cabinets (if included) | Open every door and drawer: hinges firm, doors aligned, shelves not sagging or water-damaged. |  |

# D. Balcony

Checked on each balcony, together with the wall and ceiling checks A1 to A6.

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| D1 | Railing strength | Push the railing firmly with your hands while standing back. It should not move or rattle. |  |
| D2 | Railing height and gaps | Height meets the code minimum [Confirm: BNBC figure]. Gaps no wider than about 10 cm, so a child cannot slip through. |  |
| D3 | Floor slope and drain | Pour a bucket of water: it should flow to the balcony drain, never towards the room door. |  |
| D4 | Step at the room door | The balcony floor sits lower than the room floor, so rain cannot flow inside. |  |
| D5 | Leaks into the room | Check the room wall and floor next to the balcony door for damp patches or stains. |  |
| D6 | Drain pipe | The balcony drain is piped away and does not spill onto the wall or the balcony below. |  |
| D7 | Fittings | Light point, drying rod and any grill or net in the agreement are installed and working. |  |

# E. Whole flat

Checked once for the whole flat.

## Electricity

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| E1 | Distribution board (DB) | Easy to reach, cover on, every breaker labelled, and none trips when switched on. Do not open the cover. |  |
| E2 | Electricity meter | Switch off your main breaker and ask the staff to confirm your meter stops recording. Note the meter number. |  |
| E3 | Earthing | Ask how the flat is earthed and for the earth test result. Three-pin sockets must have a working earth. [Confirm] |  |
| E4 | Generator backup | Ask for the generator to be run. Check which lights, fans and sockets get backup, as the agreement promises. |  |
| E5 | IPS provision (if promised) | A separate IPS wiring point or circuit where agreed. |  |

## Water

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| E6 | Water source and quality | Ask whether water comes from WASA, a deep tubewell or both. Run the taps and check colour, smell and sediment. |  |
| E7 | Hidden leaks | Close every tap and listen. Running water sounds, or drips under sinks, mean a hidden leak. |  |

## Entrance and area

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| E8 | Main entrance door | Solid, closes and locks firmly, with the peephole, safety chain and stopper the agreement lists. |  |
| E9 | Doorbell and intercom | The doorbell rings, and the intercom connects to the reception or guard room both ways. |  |
| E10 | Flat area | Ask how the sold area was calculated (with or without a share of common space), and compare main room sizes with the plan. [Confirm] |  |

# F. Building and common areas

Checked once. Many of these are shared, so the buyer is checking that what the agreement promised has been delivered.

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| F1 | Lift ride | Ride to every floor: smooth start and stop, floor level with the landing, and a working alarm button, light and fan. |  |
| F2 | Lift papers and rescue device | Ask for the installation certificate and service contract, and whether it has an automatic rescue device (ARD) for power cuts. |  |
| F3 | Stairs | Handrails firm along the whole stair, steps even, and normal and emergency lights working. |  |
| F4 | Fire exit and fire safety | Exit route clear and not locked, fire extinguishers in place and within date, and the fire alarm working if installed. |  |
| F5 | Parking | Your space is marked with your flat number as in the agreement, and your car fits the ramp and headroom. |  |
| F6 | Roof | Roof drains clear, water tank lids locked, parapet safe. Top-floor flats: check your ceiling for damp patches. |  |
| F7 | Water tanks and pump | Overhead tank and underground reservoir clean and covered; the pump starts and stops on its own. |  |
| F8 | Corridors and lobby | Lights work, and floors and walls are finished as the agreement says. |  |
| F9 | Security | Main gate, guard room and CCTV working as promised. |  |
| F10 | Rainwater around the building | Rain pipes run down to the drain, and the ground floor and parking do not hold water after rain. |  |

# G. Papers and keys

Collected at the end of the visit, before signing anything.

| Code | Check | How to check | Your note |
| --- | --- | --- | --- |
| G1 | Handover letter | Signed and dated, showing the flat number, parking number and the list of keys handed over. |  |
| G2 | Keys | Keys for every lock (main door, rooms, bathrooms, windows, letter box, meter box), counted against the list. |  |
| G3 | Defect repair period | In writing: how long the developer will repair defects after handover, and how to report them. |  |
| G4 | This report acknowledged | Give the developer this report and get it signed or acknowledged, with a repair date. |  |
| G5 | Utility papers | Electricity meter papers, gas connection papers (if any) and water connection details. |  |
| G6 | Warranties | Warranty cards for sanitary fittings, geyser, lift, generator and any appliances provided. |  |
| G7 | Layout drawings | Electrical and plumbing drawings, so future drilling does not hit hidden pipes or wires. |  |
| G8 | Approvals | A copy of the approved plan from RAJUK (or the local authority), and the status of the occupancy certificate. [Confirm] |  |
| G9 | Final payment statement | All payments and adjustments listed, with nothing outstanding. |  |
| G10 | Registration | If not done yet, the date by which the flat and its land share will be registered in your name, in writing. |  |
| G11 | Owners' association | Who runs the flat owners' association, the monthly service charge, and what it covers. |  |

# Decisions for you

- Severity levels: keep Minor, Major and Safety, or rename them?

- Report footer: every report a buyer sends reaches a developer, so a line such as "Made with [app name], a free tool by [your name]" with your website or contact would market you quietly. Include it?

- App name: the working title is "Handover Check". Do you have a name in mind?